Proofer

# Service Manual

- Technical Parameters

Size：1000*870*2000mm

Weight: 100KG

Capacity: 32trays or 40 trays each time.

- Function

This proofer is used for fermenting dough for various bread.

- Notice for Installation

1)  Installation and debugging must be operated by the the qualified electricians who have the electric certification promulgated by related official department.

2)  Make sure that the power voltage is coincide with the one signed in the label. Check whether the electrical wire is under good condition ( the power cord should be fixed up to the switch with leakproof and overload protective device.

3)  Do not pull the power cord heavily to avoid the wire falling off, which may cause leakance or electric shock.

4)  This equipment is not suitable places where are wet and under the sunlight direct radiation. Besides, avoid putting this equipment in the places around the corrosive goods and gas; avoid putting the combustible materials near this equipment.

5)  Place the equipment against the wall no less than 20cm. The ambient temperature condition: - 10℃ to 40℃, and the relative humidity condition: ≤85%.

6) In the transportation, notice the following issues: carried carefully, avoiding strong vibration; keep upright; the packed product should be stocked in the well-ventilated warehouse with the relative humidity below 85% where is no corrosive gas, no strong sunlight; avoid exposure in the open air for a long time. When it needs temporary storge in the open air, it must take rainproof and other protecting measures.

4. Operation

- Connect the line before use. Make sure the input voltage do not exceed the highest working voltage.

- Turn on the power switch, the proofer starts to work. Close the power switch, then the proofer stops. Notice: make sure to close the power source of the proofer main room after finishing using for safety.

- Lighting Control: after turning on power source, turn on the switch “LIGHT”, lighting lamp works; close the switch “LIGHT”, lighting lamp goes out; power source is turned off, lighting lamp also goes out

- Heating Working Mode: The heating controller will output voltage when the temperature is lower than the preset temperature (2℃); the heating controller will stop outputing  when the temperature reaches the preset temperature.

- Increasing Humidity Working Mode:  humidifier controller will output voltage when the humidity is lower than the preset humidity (3%RH); humidifier controller will stop outputing only when the humidity reaches the preset humidity.

- Press the key “△”or“▽” on the temperature controller to set the temperature for fermenting. then press the key “ENTER” on the left to finish the setting. The highest temperature is 50℃.

- Humidity Control:  humidity can be setted on the hair humidity controller. Humidity value has been generally setted before leaving the factory. If it is needed, turn the manual turn-knob on the hair humidity controller to adjust the appropriate humidity.

- Time control: turn on the “TIME” switch, the timer is ready for setting times. the key “△”or“▽” to add or reduce the fermenting time. Then press the” START/STOP”, The timer starts to work. Close the switch””TIME’, then the timer will stop.

- The proofing temperature should not be excessively high, or the too-high temperature will affect the saccharomycetes to reproduce normally, even it may cause the saccharomycetes to death.

- The user will be responsible for the expense of the maintaining and replacing components, which caused by operating avoiding the instruction.

Warning: Please pay attention to safe using. Please make sure the earth line is right connected. Please make sure to install the filter to the proofer. Please make sure a flexible tube is connected with the waterspout under the proofer to let the water out.

5. Routine Maintenance

1) Clean up the proofer carefully after use; Keep the box clean.

2) Wipe the proofer with cleaning fluid, avoiding the corrosive lipquid. After that, use the cloth to dry it. Prohibit to flush it directly by water.

3) Unplug the power cord for safety if long time not to operation.

Notice: All mantenance must be done after cutting off the power cord.

Electric Diagram (See Annex hereto)