### Preface

Thank you for choosing RONGMAI Baking-Fermenting Oven. Since the establishment, the Company takes “quality service, highest efficiency and best reputation” as its purpose. Adhering to the operation philosophy of “small profits but quick turnover,” the Company adopts practical work style and scientific management system. As a result, its high-quality products and services have won recognition and praise among the baking industry. The excellent performance of RONGMAI machine series as well as the all-around after-sales services will serve as a steady foundation for your success.

Thanks to the 200-odd sales outlets around the country, the Company have developed cooperative relationship with more than 1000 enterprises, and the export volume of the product also increases year by year. Depending on its high quality, the RONGMAI products, including 48 kinds of machines under 10 categories, have occupied a high standing in Chinese food industry.

With the unique design and advanced technology, the RONGMAI Baking-Fermenting Oven series has excellent performance and is easy to operate.

For the purpose of bringing the performance of this machine into full play, please read this Service Manual carefully before using this machine.

1. Meaning of Model of HODA Gas Oven

RM  C – 306 M

Company code

Gas type

Deck oven

Quantity of layer and tray

102 – 1 layer 2 trays

204 – 2 layers 4 trays

306 – 3 layers 6 trays

309 – 3 layers 6 trays

Notice:

- When placing an order, please state the model clearly, and provide the specification of baking tray if possible;

- Unless otherwise specified, the specification of baking tray is 400X600;

- Steaming function can be added if it is ordered in advance; and single layer with steaming function or all layers with steaming function is available as a choice.

  - Technical Parameters

| Model | RMC-204M | RMC-306M | RMC-309M | RMC-312M | RMC-315M |
| --- | --- | --- | --- | --- | --- |
| Specification | 2 layers 4 trays | 3 layers 6 trays | 3 layers 9 trays | 3 layers 12 trays | 3 layers 15 trays |
| Temperature(℃) | normal temperature – 300℃ |  |  |  |  |
| Voltage | uniphase 220V50Hz |  |  |  |  |
| Power | 0.2kw | 0.2kw | 0.2kw | 0.4kw | 0.6kw |
| Dimension(mm) | 1340·1000 ·1960 | 1340·1000 ·1960 | 1740·1000 ·1960 | 1700·1300 ·1960 | 2085·1500 ·1960 |
| Weight | 530kg | 650kg | 800kg | 950kg | 1150kg |

3. Application and Features

- Application

It can be applied to bake all kinds of bread,cake, Chinese dim sum,biscuit and roast chicken, roast duck, roast fish and other kinds of meat; besides, it can be used as drying machine. This oven is perfectly fit for both all food enterprises and live baking shop.

- Features

  - Easy to operate, adopting a set of advanced digital temperature controller system with litte error; baking temperature can be setted in a range from normal temperature to 300℃.

  - The chamber is designed according to the thermal allocation principle and radiation effect; it adopts the blower to ignite , which blows gas and air in best proportion into the burning pipe to combust;  the top and bottom burning pipes can control and adjust the temperature separately to make the baking productions delicious and attractive.

  - For the sake of safety, this oven is equipped with flameout protective system. When the flame is accidently out for breakdown or gas burning out, the fire ignition closes the gas source automatically in a very short time and meanwhile the alarm sounds for warming.

  - Equipped with transparent windows and lighting lamp to easily check the productions’ baking situation and effect when baking.

  - Equipped wih a set of high-quality thick thermal-insulating layer, which make this oven has excellent thermal-insulating fuction.

4.  Installation

Place the oven in the ventilating place. Fix the metal tube connecting to outside at the back of the oven’s gas-discharging entrance, which can efficiently discharge the oil smoke to outside on baking. Beside, install the exhaust fan in the suitable place to make the air circulation inside and prevent to be poisoned by carbon monoxide.

- The power supply plug should be inserted in the 220V special-purpose socket with function of leakage-proof and overload protecting device.(voltage between 220V-230V)

- Connect the LPG (gas bottle-reducing pressure vale-rubber tube-baking oven). Keep 2.5m distance between gas bottle and oven. Make sure the security of the gas transition area to prevent leaking (use the soapy water to test it before using).Don’t put the dangerous and flammable stuffs round the oven.

- Keep the right side of oven above 300mm distance against the wall, and the back and left side should be kept 200mm against the wall. Change the tube and vale for safety at least once per year.

Remark: In such a high-temperature working environment, it is prohibited to use the plastic exhaust tube, (0150 Aluminum exhaust tube on suggesting). The manufacturer is not liable for any damage arises from disoperation.

5. Operation

- On the display board, “OFF” or “O” stands for power disconnecting; “ON” or”I” stands for power connecting.

- Firstly, turn on the switch of the power supply and LPG. Secondly set the controller of the display board to “O”. Thirdly, turn on each layer’s power supply switch. For the safety of first ignition, open the door of the oven in advance. After finishing ignition, you can close the door of the oven.

- Clockwise rotate the thermostat knob to setup the temperature. When the heating pilot is glowing into green, the oven is starting heating. When the set temperature is reached, it may extinguish the fire automatically, and the red light will be on which explains the heating is stopped. As long as the temperature of the oven inside is getting lower than the set temperature, the oven will start igniting again and keep it constant. Continuously repeating the process like this, the oven can maintain the set temperature.

- After setting the temperature, the oven still fails to be ignited over three times, which will causes that the Solenoid valve stops, an alarm is given out and the red light glows. In this case, you should open the door of the oven and turn off the power supply on the thermostat display board. After 30 minutes later, try to turn on the power supply and check it out the ignition.

- After ignition, inspect the flame.

  - If the flame is light green, it shows normal and burning fully.

  - If the flame is red and dark green, it means the burning is not so fully.

  - If the flame is moving, it shows the wind is too strong.

Remark: If the flame is showing abnormal, you should adjust the wind making machine till the frame is getting normal. The suitable wind delivery can bring the perfect effect of the frame and baking.

- If you choose the automatic temperature control function, turn the thermostat knob to set the baking temperature. Timed baking is also available. You may set the baking time according to requirements. When an alarm is given out, the oven stops heating. Turn the timing switch to “OFF”, the alarm stops.

- Pulling the tension bar is to open the exhaust port. Pushing the tension bar is to shut the ventilating entrance.

- Finish baking, rotate the thermostat knob and adjust the temperature back to “O”. Remember to turn off the power supply of the oven and LPG for safety before leave.

Notice:

- This machine is just suitable for using liquid petroleum gas. For the reason of using fire to heat up, it is prohibited that using oil for baking and frying foods to prevent the fire accident caused by the hot oil. The manufacturer is not liable for any damage arises from disoperation. Do not lie or invert to lay the bottle of the LPG.

- Do not invert to lay the bottle of the LPG.

- Do not use the mid-compressing valve.

Special remark: Adjusting the size of the exhaust port can help the combustion of the baking fire.

6. Maintenance

- Close the power source and gas source after use every time; then open the air extractor to get rid of the oil smoke and waste gas.

- Keep the oven clean, both outside and the baking chamber. Clean the residue, stain and grease in the baking chamber with rag before use or after its temperature falls down once use for health and long time use.

- Check the power wire and the LPG’s pipes frequently. When finding breakage or crack, change it immediately.

Warning: It is prohibited to take water or corrosive liquid to clean the oven.

7. Circuit Diagram (See Annex hereto)

Please specify the qualified professionals to do the maintenance to avoiding accident.

If any question, please feel free to contact us.

8. Breakdown and solution

| Breakdown | Reason | Solution |
| --- | --- | --- |
| The fire goes off in 10 sec. after ignition, and the alarm is working. | 1. The induction needle is in wrong direction. 2.The induction needle breaks. 3. The igniting controller is broken | 1. Regulate the needle to keep it 3-5mm from burning pipe. 2. Change a new one. 3. Exchange to use the normal igniting controller, If the process turns to normal, the igniter should be replaced. |
| The fire can not be ignited, but there’s a little spark and a smell of the gas. | 1. The air is overflow,especially in cold days. 2. The LPG is used out or lack of pressure. 3.Pressure-reducingvale,solenoid valve or the copper nozzle is blocked by something. 4. The volume of liquefied gas is not enough. | 1. Adjust the fan to control the wind till the fire turns normal. 2. Change the gas bottle or soak the LPG’s bottle in hot water around 80C. 3. Take apart the reducing valve, solenoid valve and the copper nozzle to clean. 4. Enlarge the airflow. |
| The flame is out and there is alarming during baking | 1. The LPG air pressure is not enough or been used out. 2. Electricity is not well connected. 3. The igniter is damaged. 4. The fan doesn’t work. | 1. Change the gas bottle or soak the LPG’s bottle in hot water around 80C 2. Connect well the electricity. 3. Exchange to use the normal igniter. If it turns normal, the old should be replaced. 4. If finding the flame wide and yellow, replace the fan with a same size new one. |